| Converged Communication Systems, LLC. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | 1619 Jenks St |  |  |  |  | ##### Feature Tips Provided By |  |  |  |  |  |  |
|  |  |  |  |  |  | Suite 200 |  |  |  |  | Converged Communication Systems, LLC. |  |  |  |  |  |  |
|  |  |  |  |  |  | Evanston, IL 60201 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | Main: | 847-424-1500 |  |  |  | Avaya / Lucent Merlin Legend or Magix |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | #### Phone System |  |  |  |  |  |  |
|  |  |  |  |  |  | Fax: | 847-424-1544 |  |  |  |  |  |  |  |  |  |  |
|  | For Service & Support: |  |  |  |  |  |  |  |  |  | For further inquiries/info Support@convergedsystems.com www.convergedsystems.com |  |  |  |  |  |  |
| 847-424-1500 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | How to Verify Call Coverage (ie to Voice mail, etc…): - Goto Main Console (aka Receptionist Phone). This phone is either an MLX 20L or 4424 LD digital Set. If you do not have a main console phone, you can also use WinSPM administration software (see support@convergedsystems.com for more details). - Press MENU. - Press SysProgram. - Press Start (or Exit). - Press Extensions. Then press > (or more). - Select Group Cover. There are 1-30 Group Cover groups. Typically, 30 is used for ALL extensions that need to be covered to voice mail after a number of rings –(defaulted at 4 rings). How to add an extension to group cover: - Select the desired Group Cover Group (ie. 30). - Enter the extensions you would like in the group. Then hit Enter. - You have successfully entered an extension into a Cover Group. Hit Exit to get out of system programming. And make a test call to that extension to see if it covering properly. If not, see support@convergedsystems.com. To Verify What Extensions are include in Cover Group: - You can verify the extensions in each particular group by going back to Group Cover. Enter the Group you are interested in checking (ie. 30). - Select Inspect button. The display should have a list of extensions in the group. Use the > (or more) key to scroll through. To locate Inspect button, see buttons on Receptionist console phone. Usually, an Inspect button needs to be programmed like a line appearance button – and if you do not see this button then you will not be able to Inspect. See support@convergedsystems.com for more details. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |